----- RHABSIM FILE CONVENTIONS -----

RHABSIM comes with a number of files and will create files during its

operation. This document file describes each file, what it contains, its

purpose, default filenames and extensions and which RHABSIM module reads or

writes it.

RHABSIM files are of three types:

1) RHABSIM program files (executable files, configuration files and help

text files),

2) Binary data files (cross-section and calibration parameter data), and

3) ASCII text files (reports, tables and exported spreadsheets.)

The supplied RHABSIM program disk has a subdirectory named SAMPLES which

contains demonstration data files and some typical ASCII text report files.

-------------------------- RHABSIM PROGRAM FILES -----------------------------

R.BAT        - batch file that runs INTRO.EXE and FIELDAT.EXE

INTRO.EXE    - Title screens and copyright notice.

TRPx.VGX     - INTRO graphics files.

RHABSIM.CFG  - configuration file that stores default input/output file path,

units of measurement and color scheme for all RHABSIM

programs except CRITERIA. If the file is missing, one is

created using the current directory.

FIELDAT.EXE  - Cross-section data building program.

FIELDAT.HLP  - Text file displayed from the FIELDAT Help Menu.

RHB3D.EXE    - 3-D graph subprogram run by FIELDAT.

RHBFXS.EXE   - Cross-section insert/delete subprogram run by FIELDAT.

RHABSIM.MTR  - Default Velocity Meter data file storing meter names and

revs/time to velocity formulas. This file can be edited in

FIELDAT and alternate .MTR files saved and loaded.

CONVERT.EXE  - Import/Export RHABSIM data files to PHABSIM/ASCII files.

Convert units of measurement.

CONVERT.HLP  - Text file displayed from the CONVERT Help Menu.

HYDSIM.EXE   - Hydraulic calibration program. Calibrations of WSLs and

Velocities.

HYDSIM.HLP   - Text file displayed from the HYDSIM Help Menu.

RHBMSQ.EXE   - Channel Conveyance model subprogram run from HYDSIM.

RHBREG.EXE   - Log/Log Regression model subprogram run from HYDSIM.

RHBWSP.EXE   - Step-backwater model subprogram run from HYDSIM.

RHBWSP.HLP   - Text file displayed from RHBWSP Help Menu.

RHBPLT.EXE   - Graph plotting subprogram run from HYDSIM.

CRITERIA.EXE - Criteria/suitability curve library program.

CRITERIA.CFG - configuration file storing default input/output data paths. If

the file is missing, one is created using the current directory.

CRITERIA.HLP - Text file displayed from the FIELDAT Help Menu.

HABSIM.EXE   - Habitat modelling (Weighted Usable Area) program.

HABSIM.HLP   - Text file displayed from the HABSIM Help Menu.

RHBRUA.EXE   - WUA results comparison subprogram run from HABSIM.

RHBRUA.HLP   - Text file displayed from RHBRUA Help Menu.

*.LST   - ASCII list of .RUA files to load into RHBRUA. These list files

can be created in RHBRUA or in an ASCII text editor.

RHBHBG.EXE   - Graph plotting subprogram run from HABSIM.

TIMESER.EXE  - Time Series, Flow and Habitat duration analysis.

TSOVLY.EXE   - Graph plotting subprogram run from TIMESER.

*.RTS   - Binary data files for TIMESER.

----------------------- RHABSIM BINARY DATA FILES -----------------------------

.RHB files - These are data files used by RHABSIM. They contain the

cross-section data as well as calibration and modelling parameters and

coefficients. RHB files are read by all RHABSIM programs except CRITERIA.

.RSR files - These files are created when CONVERT Imports a PHABSIM data

file or prepares to Export to a PHABSIM file. Data stored in them are many

PHABSIM parameters (IOC strings and data not otherwise used in RHABSIM.) RSR

files allow you to Import one PHABSIM file format, change some parameters and

re-export to a different PHABSIM file format while still retaining the

original information for the original format. If CONVERT cannot find a

corresponding RSR file for a RHB file, a blank one will be created.

CONVERT.EXE is the only RHABSIM program which uses .RSR files. If you are not

interested in saving imported IOC strings and miscellaneous data from PHABSIM

files, you may delete the .RSR file(s) that CONVERT creates.

.RRW files - These files contain raw field information used to calculate

bottom profiles (Rod and depth readings) and velocities (meter types, revs,

times and angles). This data is only editable in FIELDAT.EXE but CONVERT will

create a blank .RRW file whenever creating a .RHB file. Please note that .RRW

files can be very large (a 50 cross-section, 100 pts per cross-section, 5

velsets per cross-section data file will create a .RRW file larger than 1

million bytes.) FIELDAT.EXE is the only RHABSIM program that uses RRW files.

If you are short on disk space and you are completely finished with

cross-section building in FIELDAT and ready to proceed to HYDSIM and HABSIM

you may safely delete your .RRW file. The RRW file only stores the data used

to *calculate* bottom profile and velocities (rod, depth, rev, time readings,

etc.), the actual final coordinates/vels are stored in the regular .RHB file.

.RCV files - These are Criteria/Suitability Curve library data files

created in CRITERIA and used for Weighted Usable Area calculations in HABSIM.

.RWL files - These contain Calibration Flows (CalQARDs) and WSLs for the

Channel Conveyance and Step-backwater models. RWL files are created in HYDSIM

when calibrating these two models and are used in HYDSIM and HABSIM. The .RWL

file and corresponding .RHB file must be located in the same directory for

HABSIM to be able to find it and use Channel Conveyance and Step-backwater

reults in HABSIM.

.RJB files - HABSIM Job Files are created and used in HABSIM. RJB files

store names and locations of .RHB and .RCV files as well as "Production Run"

parameters. As archives, RJB files allow you duplicate a run at a later time.

.RUA files - These files contain the results of Weighted Usable Area

calculations from a Production Run in HABSIM. RUA files and corresponding .RJB

files are used in RHBRUA to compare results of different jobs (production

runs.)

.RQA files - Binary data files storing flows and Weighted Usable Area data.

Used by RHBHBG program to avoid recalculating WUA for graphs.

.RTS files - Binary data files for TIMESER.EXE Time Series and FLow and

Duration analysis program. Data includes streamflow matrix, Weighted Usable

Area criteria curves and operational parameters.

.TP3 and .TP4 files - HYDSIM will export a calibrated .RHB file to the old

PHABSIM "TAPE3" and "TAPE4" binary file formats. ".TP3" and ".TP4" are the

default extensions for these filenames.

------------------------ RHABSIM ASCII TEXT FILES ----------------------------

Most of the RHABSIM programs write reports and tables to ASCII text files.

Each routine writes to a different default filename. If that filename already

exists (you have run the routine before) you are usually prompted for

filename.

The text files discussed below are arranged by which RHABSIM program creates

them:

----- FIELDAT Report and Table Text Files -----

CALIB.TXT - Reports Calibration Stage, Given Flow and Calculated Flows for

each Cal Set at each cross-section.

HY-INFO.TXT - Reports hydraulic parameters (wet count, wetted ares, wetted

width, hydraulic radius, and average depth) for your Calibration Sets (Given

Stages and Flows.) This is basically the same report as HY-CAL.TXT from

HYDSIM.

PROBLEM.TXT - Reports information on cross-section and calibration data.

Identifies problems with data sets, i.e., points under water but with no vel

readings, points out of water but with velocity readings, etc.

SUMMARY.TXT - Tabulates Given and Calculated Flows and averages throughout

cross-sections.

VELSUMM.TXT - Tabulates WSLs, Flows with and without angles, Average Vels

with and without angles and averages for each Calibration Set.

XSDATA.TXT - Reports X and Y coordinates, Velocities and Attributes for

each point at each cross-section.

----- HYDSIM Report and Table Text Files -----

CAL-SUMM.TXT - Reports all Calibration parameters for your data file,

including WSL and Vel methods. This is a record of all parameters and methods

used.

HY-CAL.TXT - Reports hydraulic parameters (wet count, wetted ares, wetted

width, hydraulic radius, and average depth) for your Calibration Sets (Given

Stages and Flows.) This is basically the same report as HY-INFO.TXT from

FIELDAT.

HY-FLOW.TXT - Similar to HY-CAL.TXT except reports hydraulic parameters for

each Calibration Flow and calculated WSL.

IFG-REG.TXT - Reports regression statistics for the Log/Log Regression

WSL Model, including A & B coefficients, percent mean error, variance,

standard deviations and ratios of Given to Predicted Flows.

S-B-REG.TXT - Similar to IFG-REG.TXT except for Step-backwater regression

WSL Model.

VAF.TXT - Table of WSLs, Calibration Flows, Calculated Flows and Velocity

Adjustment Factors for each cross-section.

VEL.TXT - Table of calculated velocities for each Calibration Flow at each

point and each cross-section.

WSL.TXT - Tables of calculated (calibration) WSLs for one or more

calibration models at each cross-section arranged by either Calibration Flow

or by cross-section.

----- CRITERIA Report and Table Text Files -----

CV-SUMM.TXT - Summary report of Curve Sets in Curve Set Library.

CVxxxxx.TXT - Table of Curve Set data. Default name is "CV" plus first six

digits of Curve Set ID number.

----- HABSIM Report and Table Text Files -----

JOB-RPT.TXT - Reports Simulation Flows and Distances, Weights, Reach

Lengths and Chosen WSL Method for cross-sections. Summarizes Curve Sets

included in Job File.

VEL-JOB.TXT - Table of Velocities for each Simulation Flow at each point at

each cross-section.

WSL-JOB.TXT - Report on WSLs for each Simulation Flow arranged by Flow or

cross-section.

WUA-DET.TXT - Details Table of Weighted Usable Area calculations. Includes

Job and WUA summary information as well as X & Y coordinates, velocities,

attributes and criteria curve values, cell widths and areas, combined

suitability factors and usable areas for each cell at each cross-section for

each Simulation Flow.

WUA-JOB.TXT - Summary of Weighted Usable Areas (available habitat per 1000

reach distance units), simulation flows, total surface area, available area

and percent of total area for each Simulation Flow for each Criteria Curve

Set.

WUA-XS.TXT - Report and tables of WSLs, reach lengths, gross and usable

areas, gross and usable areas per 1000 reach distance units and percent

usable. This report is arranged by Simulation Flows by each Curve Set.

----------

The WUA comparison subprogram, RHBRUA.EXE writes three report files. For

ease of recognition they all start with the exclamtion point character (!).

!CV-RPT.TXT - Summary report on Curve Set(s) in RUA File List file (.LST

file.) Includes which cross-section Attribute is assigned to which Curve Set

in Curve Set library.

!RUA-RPT.TXT - Reports WSLs, gross areas WUAs and percents of gross for

each flow and each Curve Set in RUA File List file (.LST file.) Arranged by

RUA file.

!WUA-RPT.TXT - Reports Gross Area, WUA and percent of gross for each RUA

file loaded, each flow by each Curve Set. Arranged by Flow and Curve Set.

----- TIMESER Report Files -----

The Time Series, Flow/Habitat Duration analysis program writes four report

files:

!RUA-RPT.TXT - Identical to the RHBRUA text file. Reports WSLs, gross areas

WUAs and percents of gross for each flow and each Curve Set in the TIMESER

.RTS File.

TSFLOW.TXT - Table of column summaries from Streamflow matrix. Includes

minimum, maximum, mean, median and count.

TSRESULT.TXT - Results of Flow/Habitat Duration analysis.

TSMATRIX.TXT - Optional file with tables of individual flow/habitat values

corresponding to Streamflow matrix. Second option sorts values and removes

duplicates.